CALICE ECAL Readout Electronics

Conceptual Design Review

Internal interface : VME crate customisation.

We propose to use COTS VME crates and backplanes. If possible with a modular Power Supply System which will allow use to tailor the power

requirements to just that which is necessary .

Extra bussed signals required will be LVDS and manually added to the backplane (the spare 64pins on J2). Non bussed signals (Geographical

Addressing) will be added with WireWrap.

n.b.  VME already exists in all institutions and therefore expertise, crates

extenders etc. will already exist, as will some form of  host CPU (either VME CPU or VME to PCI interfaces)